NILM Connections:

Sure. Here are the simple pin-to-pin connections.

LM358 Power

- LM358 Pin 8 → ESP32 3.3V

- LM358 Pin 4 → ESP32 GND

Bias Circuit

- ESP32 3.3V → 10kΩ → Node A

- Node A → 10kΩ → GND

- Node A → LM358 Pin 3

- LM358 Pin 2 ↔ Pin 1 (connect directly)

CT (3.5 mm Socket)

Your socket mapping:

- Pin 4 = Tip

- Pin 3 = Sleeve

Connections:

- Socket Pin 4 (Tip) → 10µF capacitor → Node B

- Socket Pin 3 (Sleeve) → LM358 Pin 1 (VBIAS)

- Node B → 100kΩ → LM358 Pin 1 (VBIAS)

- Node B → LM358 Pin 5

Gain Stage

- LM358 Pin 6 → 1kΩ → LM358 Pin 1 (VBIAS)

- LM358 Pin 6 → 22kΩ → LM358 Pin 7

ESP32

- LM358 Pin 7 → 100Ω → ESP32 GPIO34

- ESP32 GPIO34 → 100nF capacitor → GND

| LCD Pin (I2C Backpack) | ESP32 Pin | Wire / Note |
| --- | --- | --- |
| GND | GND | Ground |
| VCC | VIN (5V) | LCD backlights require 5V power |
| SDA | GPIO 21 | Data line |
| SCL | GPIO 22 | Clock line |